# General License Examination – Memorization Sheet

General Class Frequency Privileges

10 meters	28000 – 29700 kHz (28300-29700 phone)

12 meters	24890 – 24990 kHz

15 meters	21025 – 21200 kHz and 21300 – 21450 kHz

17 meters	18068 – 18168 kHz

20 meters	14025 – 14150 kHz and 14225 – 14350 kHz

30 meters	10100 – 10150 kHz

40 meters	7025 – 7150 kHz and 7225 – 1300 kHz

80 meters	3525 – 3750 kHz and 3850 – 4000 kHz

160 meters	1800 – 2000 kHz

Maximum 1500 watts PEP

Except  200 watts PEP on 30 meters and Novice/Tech+ sub bands on 80 meters (3675-3725 kHz), 40 meters (7100-7150 kHz) and 15 meters (21100-21200 kHz)

60 meter rules

5 authorized channels 2.8kHz wide with USB and 50 watts ERP maximum.  No interference to adjacent services and records must be kept if gain antenna is used.

RTTY/data near center of CW allocation (170Hz shift for amateur RTTY)

80 meter data	3580 – 3620 kHz

20 meter RTTY	14.070 – 14.095 MHz	20 meter PSK31	14.070

Maximum Symbol Rate for Packet, RTTY, or Data

Below 10 meters (28 MHz)	300 baud

10 Meter band	1200 baud

6 and 2 meters	19.6 kilobaud	Maximum 20Khz bandwidth

Amplifier may require FCC certification if drive power less than 50 watts below 2 meters

General privileges can be used immediately with General CSCE by adding “/AG” to callsign on CW and “Temporary AG” on phone

Minimum Channel Separation

CW	150 – 500 Hz	RTTY	250 – 500 Hz

SSB	3 kHz

| International Telegraph Union (ITU) Regions Region 1 – Europe and Africa Region 2 – North and South America Region 3 – Asia and Australia Propagation | Power Multipliers One S-Unit = 6dB = 4 fold power change 3dB = 2 fold power change Sideband Operation Below 14 MHz use lower sideband (LSB) Above 14 MHz use upper sideband (USB) |
| --- | --- |

D layer absorbs

E layer maximum single hop distance 1200 miles at altitude of 70 miles

F2 layer maximum single hop distance 2500 miles

A two tone linearity test uses two non-harmonically related audio tones

Wire Sizes

15 amp circuit requires 14 gauge wire and 15 amp breaker

20 amp circuit requires 12 gauge wire and 20 amp breaker

Peak Envelope Power

PEP = [(0.707PEV)(0.707PEV)]/RL	Where:	PEV = Peak Voltage

RL = Resistive Load

Ohm's Law and Power Formulas

I2

R

E

I

I

R

E

P

P

E = Voltage in Volts

I = Current in Amperes

R = Resistance in Ohms

P = Power in Watts

Cover the value you need and divide or multiply the remaining values as appropriate

Examples:

E = I x R	I = E / R	R = E / I

P = I x E	I = P / E	E = P / I

P = I2 x R	R = P / I2

Antenna Lengths

| Yagi Driven Element L (Ft) = 472 F (MHz) Yagi Reflector Element L (Ft) = 490 F (MHz) Yagi Director Element L (Ft) = 458 F (MHz) Where: L is Length in Feet and F is Frequency in MHz | Full Wave Loop Driven L (Ft) = 1005 F (MHz) Full Wave Loop Reflector L (Ft) = 1030 F (MHz) Divide Full Wave Loop by 4 for one side of Quad Loop Divide Full Wave Loop by 3 for one side of Delta Loop Reflector is ~3% longer than driven element Director is ~3% shorter than driven element |
| --- | --- |

Threshold Power for Station Evaluation

10 meters – 50 watts

15 meters – 100 watts

20 meters – 225 watts

30 meters – 500 watts

30 MHz (10 meters) – 300 MHz (1.25 meters) are highest exposure risk

1270 MHz is resonant frequency of eyes